DAY 3 : PERIPHERAL TROUBLESHOOTING

TASKS :

1. List 5 common issues users face with USB keyboards or mice on laptops, and write one possible solution for each.

### 5 Common Issues with USB Keyboards or Mice & Solutions

#### (1) Device Not Recognized / Unresponsive

- Solution: Unplug the USB connector/dongle and re-insert it directly into a different USB port on the laptop (avoid unpowered USB hubs).

#### (2) Wireless Lag or Intermittent Disconnection

- Solution: Replace the AAA/AA batteries in the keyboard or mouse (or recharge it) and move the USB receiver closer to the device to eliminate interference.

#### (3) Mouse Cursor Stuttering or Jumping

- Solution: Clean the optical sensor on the underside of the mouse using a dry microfiber cloth and use a clean, non-reflective mouse pad.

#### (4) Repeating Keys or Key Ghosting

- Solution: Blow out dust and debris under the keycaps using compressed air and disable Filter Keys under Windows Accessibility settings.

#### (5) Driver Error ("Unknown USB Device")

- Solution: Open Device Manager, expand Human Interface Devices (or Mice and other pointing devices), right-click the device, select Uninstall device, and restart the PC to automatically reinstall drivers.

2. Simulate a troubleshooting flow: Your Bluetooth headphones are not connecting to your phone. Write the exact step-by-step actions you would take to diagnose and fix the issue, mentioning any settings or indicators you would check.

Here is a systematic, step-by-step troubleshooting flow to diagnose and fix a pair of Bluetooth headphones that won't connect to a smartphone.

## 1. Quick Initial Check (The Basics)

- Action: Check the battery status of both devices.

  - Indicators: Look for an LED light on the headphones/case (e.g., solid red or low-battery alert sound). Ensure the phone isn't in Low Power Mode, which can sometimes restrict wireless connections.

- Action: Check device proximity.

  - Fix: Bring the headphones within 3 feet (1 meter) of the phone to eliminate range or signal interference issues.

## 2. Verify Bluetooth Settings & State

- Action: Check phone settings.

  - Path: Open Settings > Bluetooth (or swipe down/up for Quick Settings/Control Center).

  - Check: Ensure Bluetooth is actually toggled ON.

  - Fix: Toggle Bluetooth OFF, wait 5 seconds, and toggle it back ON to refresh the radio.

- Action: Check for "Airplane Mode."

  - Fix: Ensure Airplane Mode hasn't been accidentally turned on.

## 3. Check Connection Status & Existing Paired Devices

- Action: Check if the headphones are connected to another device (laptop, tablet, TV).

  - Indicator: A solid blue LED on the headphones (instead of a blinking light) usually means it auto-connected to a previously paired device nearby.

  - Fix: Disconnect or turn off Bluetooth on all other surrounding devices.

## 4. Force Pairing Mode

- Action: Put the headphones into discoverable/pairing mode manually.

  - Indicators: Look for a fast blinking LED (often alternating red/blue or white/blue) or a voice prompt saying "Pairing".

  - How-to:

    - Over-ear/On-ear: Hold down the Power/Bluetooth button for 5–7 seconds until the LED starts flashing rapidly.

    - Earbuds: Place earbuds in the charging case, leave the lid open, and hold the case button for 3–5 seconds until the light blinks.

## 5. Clear Cache & Re-pair Devices

If the phone sees the headphones but refuses to connect (e.g., "Connection Failed" error):

- Action: Forget/Unpair the device.

  - Path: Go to Settings > Bluetooth, find your headphones under Paired Devices, tap the (i) or gear icon, and select Forget This Device / Unpair.

- Action: Scan and Re-pair.

  - Put the headphones back into Pairing Mode.

  - On the phone, tap Scan or check under Other Devices. Select your headphones when they appear and accept the pairing prompt.

## 6. System Refresh (Restart & Reset)

If re-pairing fails:

- Action 1: Restart the Phone.

  - A simple phone reboot clears temporary software glitches in the OS Bluetooth stack.

- Action 2: Factory Reset the Headphones.

  - How-to: Depending on the model, hold the Power and Volume Down buttons simultaneously for 10 seconds (or hold the case button for 15+ seconds) until the LED flashes red/white quickly. This clears all paired memory on the headphones.

- Action 3: Reset Network/Bluetooth Settings on Phone.

  - iOS: Settings > General > Transfer or Reset iPhone > Reset > Reset Network Settings.

  - Android: Settings > System > Reset options > Reset Bluetooth & Wi-Fi.

3. A user reports that their external webcam is not detected during a Zoom call on Windows 10. Write a checklist of 6 things to check or try to resolve the problem. <br><br><em><strong>Hint: </strong>Consider device manager, privacy settings, and app permissions.</em>

### Troubleshooting Checklist: External Webcam Not Detected on Zoom (Windows 10)

#### 1. Check Physical Cable Connection & USB Port

- Unplug the webcam's USB cable and reconnect it firmly to a direct USB port on the PC/laptop (preferably a USB 3.0 port on the rear motherboard panel, avoiding unpowered USB hubs or extensions).

#### 2. Verify Device Recognition in Device Manager

- Press Win + X and open Device Manager.

- Expand Cameras (or Imaging Devices / Sound, video and game controllers).

- Check if the webcam is listed. If there is a yellow exclamation mark (!), right-click and select Update driver or Enable device.

#### 3. Check Windows 10 Camera Privacy Settings

- Open Settings (Win + I) $\rightarrow$ Privacy $\rightarrow$ Camera.

- Ensure "Allow access to the camera on this device" is turned ON.

- Make sure "Allow apps to access your camera" and "Allow desktop apps to access your camera" (which includes Zoom) are both set to ON.

#### 4. Verify Video Input Settings Inside Zoom

- Open the Zoom app $\rightarrow$ Click your profile icon/gear icon to open Settings $\rightarrow$ Go to the Video tab.

- Under the Camera dropdown menu, select your external webcam explicitly (instead of an integrated laptop camera or virtual camera).

#### 5. Check for App Conflicts & Physical Privacy Covers

- Close any other applications running in the background that might be using the camera (such as Microsoft Teams, Skype, Google Meet, or the Windows Camera app), as Windows restricts webcam access to one app at a time.

- Inspect the physical webcam lens to ensure any hardware privacy shutter/cover is fully opened.

#### 6. Uninstall & Reinstall Webcam Drivers

- Open Device Manager, right-click your external webcam, and select Uninstall device.

- Unplug the webcam, restart your Windows 10 PC, and reconnect the webcam to let Windows automatically download and reinstall clean default drivers.

4. You are helping a friend whose printer is showing 'offline' status on their PC. Describe how you would use the Windows Troubleshooter tool to try to fix the issue, and explain what information the tool provides during the process.

Here is a detailed guide on using the Windows Troubleshooter tool to resolve a printer showing an 'offline' status, along with an explanation of what information the tool provides.

### Part 1: Step-by-Step Instructions to Run the Printer Troubleshooter

- Open Windows Settings:

  - Press Win + I on the keyboard to launch Settings.

- Navigate to Troubleshoot Settings:

  - Click on Update & Security $\rightarrow$ Select Troubleshoot from the left sidebar menu.

  - Click on Additional troubleshooters (or Other troubleshooters on Windows 11).

- Run the Printer Troubleshooter:

  - Under the Get up and running section, select Printer.

  - Click Run the troubleshooter.

- Select the Affected Printer:

  - A window will open listing all printers configured on the PC. Select the printer displaying the 'offline' status and click Next.

- Apply Recommended Fixes:

  - The tool will scan for common issues. If it finds a problem (e.g., Print Spooler stopped, port mismatch), it will display a prompt asking to "Apply this fix". Click to apply the suggested solution automatically.

### Part 2: What Information the Tool Provides During & After the Process

During the diagnostic run, the Windows Troubleshooter continuously scans system services, network protocols, and drivers, providing key information at each phase:

- Diagnostic Progress / Status Updates:

  - Shows real-time progress steps such as "Checking Print Spooler service status...", "Checking network connectivity...", and "Checking device drivers..." so you know which subsystem is being tested.

- Root Cause Identification:

  - Displays the exact reason why the printer was appearing offline. Common findings include:

    - Print Spooler Service is stopped or unresponsive.

    - Printer default status is set to "Use Printer Offline".

    - Corrupt or missing printer driver.

    - IP address or port mismatch for network printers.

- Automated Action / Fix Summary:

  - Tells you what actions it took to resolve the issue (e.g., "Restarted the Print Spooler service", "Cleared pending print queue jobs", or "Set the printer as default").

- Final Summary Report:

  - After completion, the tool provides a summary report categorizing detected issues as:

    - Fixed: Issues that were successfully diagnosed and resolved automatically.

    - Not Fixed / Unresolved: Issues that require manual intervention (e.g., physical hardware disconnected, Wi-Fi network down, or needing custom vendor drivers).